REQUEST FOR PROPOSAL
SPECIFICATION OF WORKS

Procurement No:	21-W006-20

## Specification

### Background

The sea cucumber hatchery in Tanaea was built in the 1995 by the Government of Japan through the Overseas Fisheries Cooperation Foundation (OFCF) project. The aim of this project is to carry out restocking program for sea cucumber to ensure the wild stock population is maintained to support fishermen.

The target species this for the project was the white teatfish (Holothuria fuscogilva) due to heavy fishing pressure from local fishermen due to its value. Successful spawning commences in 1997 led by an OFCF expert who carried out spawning training program to the local staff. Over time, the hatchery started expanding its target species focussing on giant clams and trouchus for the ornamental trade.

More than 20,000 white teatfish juveniles were released in Tarawa and Abaiang lagoon.

When the project ended in 2001, the Kiribati Government continue to support the project activities. The scarcity of white teatfish broodstock shift the target species to sandfish (H. scabra). Since sandfish is not native to Kiribati, it was introduced from Fiji. Sandfish is also a very high valued species and is more suitable for breeding as it is tougher and much easier to handle compared to white teatfish.

Sandfish was introduced in 2013 followed by two more consecutive re-introduction in 2014 and 2015. The spawning was very successful followed by community farming in Abaiang and North Tarawa. The growth rate of sandfish was very promising and shows that sandfish farming has a potential in Kiribati from the hatchery reared seeds.

In 2017, the Kiribati Government managed to extend the hatchery by constructing additional rearing tanks areas which handles about 20 raceways. This has enable the hatchery to carry out sandfish and giant clam spawning at once without the concern of space for juveniles.

For more than 24 years since the hatchery was first built, it continues to produce sandfish and giant clam seeds, however age has taken its toll affecting the overall performance of the hatchery. With growing number of the hatchery staff, the working space inside the hatchery is now a little bit crowded and need to be expanded. The condition of the lap and the algae culture room is not very suitable for culturing live feed algae and as such an overall upgrading to the hatchery is needed.

### Requirements

Please refer to the “Instructions on how to submit the tender templates” for a list of requirements for eligibility and exclusionary crteria

For the purpose of evaluating the tender submissions, the following criteria will be considered

- Work schedule with a clear delivery timeframe for delivery of materials and the duration of the construction work

- Confirmed list of the available working tools

- References from past year experiences in the similar construction work and supply of materials

- A clear pricing quotation that is complete (i.e, inclusive of relevant tax, freight for importing materials, etc)

- Valid business license

### Related services

N/A

### Project Time & Final Delivery

Construction materials expected to arrive after 2 weeks and maintenance work within ___ days; however, Tenderers will be assessed based on their actual and honest delivery timeframe

Description of the Works

Here, list all items to be Tendered

(This part may be replaced by a Procuring Entity or proprietary Contractor description)

SCOPE OF WORK :

Hatchery #1

- Pour new concrete slab for the entire nursery (Hatchery)

- Clean all inlet/supply pipes connection to container. Entire inlet system is require a major clean up.

- Install new roof timber member, such as rafter, purlins, etc.

- Install new roof lining as indicated on the roof RRMG plan

- Provide new channel drainage as shown on detial “Z”

- Existing PVC posts can be reused unless otherwise.

- Repair Overhead tank – Flooring and post.

Hatchery #2

- Pour new concrete slab for the entire nursery (Hatchery)

- Clean all inlet/supply pipes connection to container. Entire inlet system requires a major clean up.

- Install new roof timber member, such as rafter, purlins, etc.

- Install new roof lining as indicated on the roof RRMG plan

- Provide new channel drainage as shown on detial “Z” (consult client)

- Existing PVC Posts can be reused unless otherwise.

- Allow renovation to over tank located between Hatchery #1 and Laboratory.

LABORATORY EXTENSION

- Locate the algae room as shown on the layout and build the new extension as shown on the layout (consult client)

## Tenderer’s References

### Relevant similar deliveries carried out in the last five years

Please, provide information on each delivery for which your firm/entity, either individually as a corporate entity or as one of the major companies within an association, was legally contracted.

| Works delivered | Reference | Contact details | Value |
| --- | --- | --- | --- |